Meadow Lake Airport

AWOS Selection Analysis

By Ron Lee

17 December 2008

Table of Contents

Purpose...................................................................................................................................3

Executive Conclusion.............................................................................................................3

Background.............................................................................................................................3

Discussion...............................................................................................................................4

System Maintenance...............................................................................................................6

Federal Database Connectivity...............................................................................................7

Cost Comparison.....................................................................................................................7

AWOS System Specification..................................................................................................8

AWOS Comparison................................................................................................................8

Conclusion...............................................................................................................................9

What can you do?....................................................................................................................9

List of Tables

Table 1:  AWOS Numbers in 25 Selected States...................................................................6

Table 2:  Belfort (Digi WX) versus Vaisala Life Cycle Cost	................................................7

Table 3:  Belfort Digi WX versus Vaisala Characteristics Comparison.................................8

Table 4:  Belfort Installation Info...........................................................................................10

Table 5:  Colorado AWOS/ASOS Installations by Type.......................................................11

Purpose:  This paper documents my analysis of the available AWOS systems and provides a clear basis on which to determine the best system for Meadow Lake airport.  It also recommends threshold and objective system performance levels.

Executive Conclusion:  Of the four candidate AWOS manufacturers, only two meet the threshold and objective system performance levels.  These are systems provided by All Weather Inc (AWI) and Vaisala.  The SuperAWOS by Potomac Aviation does not pass an initial cut due to maintenance problems encountered with their SuperUnicom installed at Meadow Lake airport.

The Belfort Digi WX “AWOS” is not a true FAA fully commissionable AWOS.  Its complete sensor suite is not certified.  Installation of this system at 00V means that we would not have our weather data (METAR) accessible in the federal database which means that Flight Service and other sites such as ADDS, XM weather, WSI, etc would not not have Meadow Lake weather data.  The only certified weather info would be altimeter and visibility.  While this does allow Part 91 and 135 instrument approaches, the inability to incorporate its complete METAR data in the federal system is a deal-breaker.

The apparent initial cost savings of the initial Belfort Digi WX acquisition is minimal and not worth the lack of functionality and usefulness to the national aviation community.  Life-cycle cost of the Belfort system over 15 years is over $27,000 higher than the Vaisala system.

It would not allow us to transition to a three letter airport identifier.

Bottom line is that the Belfort system represents false economy, minimal pilot usefulness and is inconsistent with providing the aviation community services that make this airport attractive to current and future pilots and businesses.

Of the remaining systems offered by AWI and Vaisala, Vaisala is the preferred system.

Background:

The four candidate AWOS manufacturers are AWI, Belfort, Potomac Aviation, and Vaisala.  Information on their systems is available at the following websites:

1)   AWI:    http://www.allweatherinc.com/aviation/awos_dom.html

2)   Belfort:    http://digiwx.com/content/digiwx.cfm

3)   Potomac Aviation:    http://www.potomacaviation.com/index2.php

4)   Vaisala:    http://www.vaisala.com/weather/products/aviation/fornorthamerica/faaawos

The categories of AWOS are shown below.  The ones that are potential candidates for Meadow Lake airport are highlighted in bold type.

AWOS A:  Dual-pressure sensor measures barometric pressure and reports altimeter setting

AWOS A-V:  AWOS A plus visibility

AWOS I:  Wind Speed, Wind Gust,Wind Direction, Variable Wind Direction, Temperature, Dew Point, Altimeter Setting, Density Altitude

AWOS II:  Same as AWOS I + Visibility, VariabIe Visibility

AWOS III:  Same as AWOS II + Cloud Height & Sky Condition

AWOS III-R:  Same as AWOS III + Runway Surface Condition

AWOS III-P:  Same as AWOS III + Present Weather Identification Sensor

AWOS III-P-R:  Same as AWOS III-P + Runway Condition Sensor

AWOS III-P-T:  Same as AWOS III + Present Weather & Lightning Detection

AWOS III-P-T-R:  Same as AWOS III-P-T + Runway Condition Sensor

AWOS III-T:  Same as AWOS III + Lightning Detection

AWOS III-T-R:  Same as AWOS III-T + Runway Condition Sensor

AWOS III-P-T-Z:  Same as AWOS III + Present Weather & Lightning Detection, Freezing Rain Detection

AWOS IV:  Same as AWOS-P-T-Z + Runway Condition Sensor

Discussion:

Since the Belfort Digi WX has been championed by one Board member, this paper largely evaluates its usefulness.  The airports that have some Digi WX product installed (according to Belfort) are listed on their website at:

http://www.digiwx.com/content/installations.cfm

I listed the majority of the airport installations in Table 4 below.  I then looked in the Airport Facility Directory (http://tinyurl.com/4gddlu) to see if an AWOS was identified.  That is column 3 (AWOS in A/FD) in Table 1. (Note that this URL may not work after 25 September 2008.)

Column 4 (AWOS/ASOS Telephone) lists the call-in telephone number of the AWOS system if available.

Column 5 (METAR on ADDS) lists whether METAR data is available on the ADDS website:

http://adds.aviationweather.noaa.gov/tafs/

Column 6 (METAR via FSS) lists if I was able to get METAR data by calling FSS (1-800-WX-Brief).   This was a semi-random sampling plus all those reported as having an AWOS/ASOS.

Column 7 (METAR on XM) lists whether METAR data is available on a Garmin 496 with weather capability.

Column 8 (Airport opinion/Misc ) lists the opinion of the system by some airport personnel and other information.

From this table several things become clear.  Those airports listed as having Belfort installations (of unspecified nature) largely do not have three letter IDs.   AWOS is not listed as available in the A/FD.  METAR data is not available on ADDS, via Flight Service or on the Garmin 496.  In other words, airport weather info is not getting to the federal data collection system.  This may be due to the airport not being to afford the connectivity functionality...or the data is not allowed in the federal system because not allof the Belfort sensors are certified.

Looking at column three, four airports were noted as having an AWOS or ASOS in the A/FD.  These are BTM, 6S5, FFA and TKI.   6S5 is an AWOS A-V and if you call the phone number it does mention Digi WX.  If you note it, everything after the altimeter setting and visibility is ADVISORY.  In other words the data is not from a certified sensor.   The three remaining sites are clearly or almost certainly non-Belfort systems as noted in the last column.

Stooksbury Email of 10 September 2008:  In this email Mr Stooksbury of Avionics Specialists makes the following assertions in favor of the Belfort system:

Assertion 1:

“In conjunction with the publication of this memo, the FAA is working on updating the AC 150-5220-16 document on AWOS systems. It is currently revision C, but will soon (within 90 days or so) be updated to revision D. Effective with revision D, AWOS AV systems and others will be fully integrated into the FAA data system. This will mean that all data collected by the AWOS AV will be made available through standard FAA channels (FSS, METARs, DUATs, etc.) as is done with other AWOS systems like AWOS III and IV.”

My response:  There is no proof that his assertion is correct.  CDoT has stated that data from systems with certified and uncertifed sensors are not accepted in the federal database.  Even if it is correct, it may be that only the limited certified sensor data (altimeter and maybe visibility) is accepted which is hardly adequate from a pilots point of view.

Assertion 2:

“The direction is clear; the FAA wants more affordable AWOS systems at more locations rather than having fewer locations served by the high-end systems that are more expensive.

My response:  I read the program guidance and saw nothing in paragraph e to suggest that the FAA wants “more affordable” systems.  This really equates to less capable systems which is opposite of what pilots want and need.  I performed a quick analysis of the weather systems in 25 states.  The breakout is as follows and is listed in order of least capable system to most capable system.

Source data for this table is as follows:

http://www.faa.gov/AIRPORTS_AIRTRAFFIC/WEATHER/ASOS/

Or pick Colorado:

http://www.faa.gov/AIRPORTS_AIRTRAFFIC/WEATHER/ASOS/?state=CO

Table 1:  AWOS Numbers in 25 Selected States

| AWOS TYPE | Number in 25 selected states |
| --- | --- |
| AWOS A | 3 |
| AWOS A-V | 4 |
| AWOS I | 2 |
| AWOS II | 1 |
| AWOS III | 284 |
| AWOS III P | 132 |
| AWOS III T | 15 |
| AWOS III/P/T | 249 |
| ASOS | 568 |
| AWSS | 8 |

The combined percentage of AWOS A, AWOS A-V, AWOS I and AWOS II systems is about 0.8%.  That is less than ONE PERCENT.  If ASOS is roughly equivalent to AWOS III P/T then AWOS III P/T is the most prevalent system in terms of information provided to the pilot.  AWOS III is second.  Ignoring ASOS, the predominate system in Colorado is AWOS III P/T.  This is clearly shown in Table 5 below.  The reader will also note not a single AWOS A-V system such as that represented by the Digi WX is apparently in use in Colorado.

Assertion 3:

“In addition, Belfort remains optimistic that their ceilometer will be FAA certified within 90 to 120 days.”

My response:  Optimism is great.  This schedule is at best soft and at worst unrealistic.  Our airport requirement based upon pilot inputs is an AWOS III threshold and objective AWOS III P/T.  Even if Belfort gets the ceilometer certifed, the entire sensor suite must be certified and I am unaware of a realistic plan to achieving that or providing a certified AWOS III P/T system.

System Maintenance:   Meadow Lake has a great opportunity to save in yearly maintenance costs.  One person with an extensive background in aviation electronics can receive maintenance training for the Vaisala system in the Denver area for $2000.  Add in about $4,414 for test equipment and we save $3,500 annually (assumes $1,500 annually for compensation).  A similar program exists for the AWI AWOS.  It is unlikely that this will happen for the Belfort system so we would have to allocate a minimum of $3,500 annually for Belfort maintenance.

Federal Database Connectivity:  Vaisala offers an option to transmit weather date to the federal database.  This costs about $5,000 for the satellite uplink and about $150 monthly access charges.  That is unrealistic for our budget.  Fortunately, CDoT is working on an innovative program that will perform the same function at a far lower (exact number unknown) cost.  However, they cannot provide data from the Belfort system with its current suite of uncertified sensors.

Cost comparison:  Due to source selection sensitivities, actual “system only” numbers will not be included in this document.  Comparing the Belfort bid for “AWOS III” level service to the CDoT system integrator cost for the same AWOS III level system, Belfort is initially less expensive.  However, that cost differential is meaningless when you consider several factors:

1)   Our portion of that differential is minor.

2)   The entire Belfort system sensors are not FAA certifiable like Vaisala so our METAR data will not get into the federal database.

3)   Life-cycle costs make the Belfort system more costly if we cannot have our maintenance performed in-house as we can with the Vaisala (and probably AWI) systems.

4)   We have budgeted enough to get the fully certified system.

In year 2 the total life-cycle cost for the Belfort system is higher than the Vaisala option and increases by $2,000 every year thereafter.  At the end of 15 years, assuming no changes in maintenance costs, the Belfort DigiWX costs the MLAA members over $27,000 more than the Vaisala option.  This does not account for replacement/damage costs.

Table 2:  Belfort (Digi WX) versus Vaisala Life Cycle Cost

|  | Belfort Life-cycle cost MLAA Portion | Vaisala Life-cycle cost MLAA Portion | MLAA Total Cost Belfort minus Vaisala (Note 1) |
| --- | --- | --- | --- |
| Acquisition cost plus one year maintenance (Note 2) | $2,794 | $3,291 | -$497 |
| Year 2 maintenance (Note 3) | $6,294 | $4,791 | $1,503 |
| Year 3 maintenance | $9,794 | $6,291 | $3,503 |
| Year 4 maintenance | $13,294 | $7,791 | $5,503 |
| Year 5 maintenance | $16,794 | $9,291 | $7,503 |
| Year 6 maintenance | $20,294 | $10,791 | $9,503 |
| Year 7 maintenance | $23,794 | $12,291 | $11,503 |
| Year 8 maintenance | $27,294 | $13,791 | $13,503 |
| Year 9 maintenance | $30,794 | $15,291 | $15,503 |
| Year 10 maintenance | $34,294 | $16,791 | $17,503 |
| Year 11 maintenance | $37,794 | $18,291 | $19,503 |
| Year 12 maintenance | $41,294 | $19,791 | $21,503 |
| Year 13 maintenance | $44,794 | $21,291 | $23,503 |
| Year 14 maintenance | $48,294 | $22,791 | $25,503 |
| Year 15 maintenance | $51,794 | $24,291 | $27,503 |

Note 1:  If this value is positive then the Belfort system is more costly than the Vaisala option.

Note 2:  Assume MLAA pays 2.5% of total initial cost thus Belfort cost is $2794 (2.5% of $111,762) and Vaisala is $3291 (2.5% of $131,640)

Note 3:  Assumes airport rep works on the Vaisala system ($1,500/year compensation) but a Belfort rep works on that system.  This assumption has valid justification.

AWOS System Specification:

We have the budget to get an AWOS III P/T with maintenance training and test equipment, power and phones lines to the site and a good security fence around the AWOS.  If an intermediate level AWOS III system is selected AWOS III P makes the most operational sense versus an AWOS III T.  However, we have been approved for an FAA grant which makes our cost of the AWOS III P/T around $4,000.  This is an opportunity that cannot be squandered away.  An informal survey of local pilots support the AWOS III threshold and AWOS III P/T objective.

In The FAA Program Guidance Letter 08-05, Appendix A, dated July 11, 2008, Potomac Aviation (SuperAWOS) and Belfort (Digi WX AWOS) are only certified at the AWOS A and AWOS A-V level.  As of October 2007, neither company offered an AWOS III, AWOS III P/T, AWOS III P or AWOS III T system.  Thus neither company offers an AWOS that meets our minimum or objective requirements.

AWOS Comparison:     The following table compares characteristics of the Belfort and Vaisala systems.

Table 3:  Belfort Digi WX versus Vaisala Characteristics Comparison

|  | Belfort | Vaisala |
| --- | --- | --- |
| Fully certified sensor suite | NO | YES |
| Certified altimeter sensor | YES | YES |
| Certified visibility sensor | YES | YES |
| Certified winds sensor | NO | YES |
| Certified temps sensor | NO | YES |
| Certified clouds/sky condition sensor | NO (reportedly in work) | YES |
| Upgradeable to AWOS III | NO | YES |
| Upgradeable to AWOS III P | NO | YES |
| Upgradeable to AWOS III T | NO | YES |
| Upgradeable to AWOS III P/T | NO | YES |
| Allows three letter airport ID | Probably NO | YES |
| Weather info available to FSS | NO | YES |
| Weather info available to XM weather | NO | YES |
| Weather info available to ADDS | NO | YES |
| Weather info available to NADIN | NO | YES |
| Can input weather info to CDoT system | NO | YES |
| Weather available by phone | YES | YES |

Conclusion:   Of the four candidate AWOS manufacturers, only two meet the threshold and objective system performance levels.  These are systems provided by All Weather Inc (AWI) and Vaisala.  The Belfort system does not offer the performance that we need, is not fully certified and does not represent a good value for our money.  An AWOS III is the minimum system that we should procure with an AWOS III P/T desirable at initial installation or upgraded within a year.  Since neither Potomac nor Belfort offers a certified AWOS III or AWOS III P/T system, we have two choices:  All Weather Inc (AWI) and Vaisala.  I suspect that both are good but since CDoT has extensive experience with Vaisala and is satisfied with their performance, CDoT has an agreement with a system installer to use the Vaisala product, and Vaisala has a service center in the Denver area, Vaisala is the easy choice.  The closeness of the Denver area service center and nearby spares availability mean that system outages can be of shorter duration than using other systems.  Other system design factors also favor the Vaisala unit over All Weather Inc.

I will add a comment reportedly from Mr Stooksbury from around December 10, 2008:

“If you can afford it (the initial acquisition and on-going costs, including the federal tie-in), and can get the state/feds to approve it, the Vaisala system is definitely the better system. It also costs more.”

We can afford the Vaisala unit (just a few thousand with a FAA grant), we have a unique asset to reduce the typical yearly maintenance cost and the CDoT program to make access to the federal database affordable is close to fruition.  These are FACTS!

WHAT CAN YOU DO?

Email, phone, or talk in person with the Board members and tell them in no uncertain terms that you want a fully certified AWOS (AWOS III or better:  AWOS III P or AWOS III P/T).  This cannot be left to chance.  I would appreciate receiving an email stating who you contacted and a short summary of your conversation.

Ron Lee

MLAA Board member contact info

Dave Elliott		339-0928		falcon20flier@msn.com

Joe Kuberka		659-5052		Joe_Kuberka@hughes.net

Jamie Johnston	email			Jamie.Johnston@ewsys.com

Mark Shook		641-0460		mark@meadowlakeairport.com

Richard Martin	683-6587		wemartin@sprintmail.com

Ron Lee		683-3701		ronlee@pcisys.net

Table 4:   Belfort Installation Info

| Airport | ID | AWOS in A/FD | AWOS/ ASOS Telephone | METAR on ADDS | METAR via FSS | METAR on XM | Airport opinion/Misc (as of Sep 2008) General: Ceilometer not working at any airport called. ~70% like the system. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Buckeye AZ | BXK | NO |  | NO |  |  |  |
| Rio Vista CA | O88 | NO |  | NO |  |  |  |
| Greenville IL | GRE | NO |  | NO | NO | NO | Prefer Vaisala |
| Ingersoll Canton IL | CTK | NO |  | NO | NO | NO | Pilots don’t like it |
| Lake in the Hills | 3CK | NO |  | NO |  |  | Wind speed info inaccurate. System goes down during every thunderstorm. |
| Vandalia IL | VLA | NO |  | NO |  |  | Initially lots of software problems. OK now |
| Belfast ME | BST | NO |  | NO | NO | NO |  |
| Bethel ME | 0B1 | NO |  | NO |  | NO |  |
| Newton ME | 59B | NO |  | NO |  | NO |  |
| Machias Valley ME | MVM | NO |  | NO |  |  |  |
| Central Maine ME | OWK | NO |  | NO |  |  | Gives incorrect info. Want to take it out. |
| Princeton ME | PNN | NO |  | NO |  |  |  |
| Washington Executive MD | W32 | NO |  | NO |  |  |  |
| Bert Mooney MT | BTM | ASOS 135.175 | 406-494-1870 | YES | YES | YES | RICK GRIFFITH, Phone 406-494-3771. Wx recording does not sound like Digi WX. Likely not a complete Belfort system. |
| Bowman MT | 3U3 | NO |  | NO |  | NO |  |
| Edgar Obie MT | S71 | NO |  | NO |  | NO |  |
| Fort Benton MT | 79S | NO |  | NO |  |  |  |
| Libby MT | S59 | NO |  | NO |  |  |  |
| Lincoln County MT | 88M | NO. CTAF 2 clks |  | NO |  | NO |  |
| Ravalli County | 6S5 | AWOS-AV | 406-375-9149 | NO | NO | NO | PAGE GOUGH, Phone 406-375-9149. Digi WX per weather recording. Winds and temp are advisory in recording. |
| Red Lodge MT | RED | NO |  | NO | NO | NO |  |
| Ocean City NJ | 26N | NO |  | NO |  | NO | Automated Wx 3 clicks on Unicom (& by phone) Like it but would get a Vaisala next time. |
| Moriarity NM | 0E0 | NO |  | NO |  | NO |  |
| Ledgedale NY | 7G0 | NO |  | NO |  |  |  |
| First Flight NC | FFA | YES | 252-449-0698 | YES | YES | YES | Phone 252-473-2111 or 252-441-7430. AWOS IIIP. Wx recording does not sound like Digi WX. Likely not a complete Belfort system. |
| Darke Couny OH | KVES | NO |  | NO |  |  | Installed in Jan 2008. Not working yet on radio...only by phone. |
| Gen Dewill Spain TN | M01 | NO |  | NO |  |  |  |
| McKinney TX | TKI | ASOS 119.925 | 972-548-8525 | YES | YES | YES | KENNETH F. WIEGAND, Phone 972-562-4214. Only the VIS sensor is Belfort. Ceilometer is Vaisala and the main box is NOAA. Do not count as a Belfort AWOS. |
| Huntington UT | 69V | NO |  | NO |  |  |  |
| New Kent County VA | W96 | NO |  | NO |  |  |  |

Table 5:  Colorado AWOS/ASOS Installations by Type

| ID | Location | County | State | Frequency | Phone | Type |
| --- | --- | --- | --- | --- | --- | --- |
| KAKO | Akron | Washington | CO | 135.475 | (970) 345-2320 | ASOS |
| KASE | Aspen | Pitkin | CO | ATIS 120.4 | (970) 925-9168 | ASOS |
| KITR | Burlington | Kit Carson | CO | 135.225 | (719) 346-7036 | ASOS |
| KCOS | Colorado Springs | El Paso | CO | ATIS 125.05 | (719) 637-9696 | ASOS |
| KCEZ | Cortez | Montezuma | CO | 135.625 | (970) 564-0193 | ASOS |
| KCAG | Craig | Moffat | CO | 135.425 | (970) 824-2373 | ASOS |
| KDEN | Denver International | Denver | CO | ATIS 125.6 | (303) 342-0838 | ASOS |
| KDRO | Durango | La Plata | CO | 120.625 | (970) 259-3579 | ASOS |
| KAPA | Englewood - Centennial | Arapahoe | CO | ATIS 120.3 | (303) 706-9098 | ASOS |
| KGJT | Grand Junction | Mesa | CO | ATIS 118.55 | (970) 245-7881 | ASOS |
| KLHX | La Junta | Otero | CO | 135.525 | (719) 384-5961 | ASOS |
| KLAA | Lamar | Prowers | CO | 135.625 | (719) 336-3854 | ASOS |
| KLXV | Leadville | Lake | CO | 118.375 | (719) 486-2735 | ASOS |
| KLIC | Limon | Lincoln | CO | 121.125 | (719) 775-0515 | ASOS |
| KEEO | Meeker | Rio Blanco | CO | 135.525 | (970) 878-5036 | ASOS |
| KMTJ | Montrose | Montrose | CO | 135.225 | (970) 249-1534 | ASOS |
| KPUB | Pueblo | Pueblo | CO | ATIS 125.25 | (719) 948-2803 | ASOS |
| KRIL | Rifle | Garfield | CO | 135.275 | (970) 625-2206 | ASOS |
| KALS | San Luis Valley/Bergman Field | Alamosa | CO | 135.175 | (719) 589-5669 | ASOS |
| KTAD | Trinidad - Perry Stokes | Las Animas | CO | 119.025 | (719) 845-1156 | ASOS |
| KGNB | Granby | Grand | CO | 119.925 | (970) 887-1803 | AWOS I |
| KEGE | Eagle - Gypsum | Eagle | CO | 135.575 | (970) 524-7386 | AWOS III |
| KFNL | Fort Collins - Loveland | Larimer | CO | 135.075 | (970) 669-9187 | AWOS III |
| KGXY | Greeley | Weld | CO | 135.175 | (970) 352-3511 | AWOS III |
| KGUC | Gunnison | Gunnison | CO | 135.075 | (970) 641-3240 | AWOS III |
| KANK | Salida | Chaffee | CO | 133.85 | (719) 539-5268 | AWOS III |
| KSBS | Steamboat Springs | Routt | CO | 118.325 | (970) 879-7794 | AWOS III |
| K33V | Walden | Jackson | CO | 118.625 | (970) 723-4513 | AWOS III |
| KHDN | Yampa Valley | Routt | CO | 119.275 | (970) 276-3690 | AWOS III |
| K1V6 | Penrose | Fremont | CO | 120.025 | (719) 784-2014 | AWOS IIIP |
| K0CO | Berthoud Pass | Clear Creek | CO | 134.325 | (303) 512-4416 | AWOS IIIP/T |
| K1V5 | Boulder | Boulder | CO | 118.825 | (303) 541-9540 | AWOS IIIP/T |
| KBJC | Broomfield | Broomfield | CO | 126.25 | (720) 887-8067 | AWOS IIIP/T |
| KAEJ | Buena Vista | Chaffee | CO | 132.925 | (719) 395-2599 | AWOS IIIP/T |
| KCCU | Conifer | Jefferson | CO | 118.075 | (970) 968-1715 | AWOS IIIP/T |
| KAJC | Delta-Blake | Delta | CO | 134 | (970) 874-3251 | AWOS IIIP/T |
| KFMM | Fort Morgan | Morgan | CO | 132.95 | (970) 867-4823 | AWOS IIIP/T |
| KFTG | Front Range | Adams | CO | 119.025 | (303) 261-9104 | AWOS IIIP/T |
| K5SM | Glenwood Springs | Garfield | CO | 126.075 | (970) 384-3380 | AWOS IIIP/T |
| KHEQ | Holyoke | Phillips | CO | 119.275 | (970) 854-3679 | AWOS IIIP/T |
| K20V | Kremmling | Grand | CO | 118.425 | (970) 724-9659 | AWOS IIIP/T |
| K4BM | Lake George | Park | CO | 132.3 | (303) 512-4418 | AWOS IIIP/T |
| KVTP | Laveta Pass | Huerfano | CO | 119.925 | (719) 587-3120 | AWOS IIIP/T |
| K2V2 | Longmont | Boulder | CO | 120 | (303) 684-7545 | AWOS IIIP/T |
| KAIB | Nucla | Montrose | CO | 135.525 | (970) 864-2325 | AWOS IIIP/T |
| K04V | Saguache | Saguache | CO | 118.625 | (719) 655-2229 | AWOS IIIP/T |
| KMYP | Salida/Monarch Pass | Saguache | CO | 124.175 | (719) 539-5268 | AWOS IIIP/T |
| K3MW | Steamboat Springs | Routt | CO | 127.125 | (970) 871-5640 | AWOS IIIP/T |
| KSTK | Sterling | Logan | CO | 118.525 | (970) 526-3009 | AWOS IIIP/T |
| KTEX | Telluride | San Miguel | CO | 118.325 | (970) 728-1534 | AWOS IIIP/T |
| KMNH | Watkins | Adams | CO | 134.375 | (303) 648-3653 | AWOS IIIP/T |
| KCPW | Wolf Creek Pass | Rio Grande | CO | 121.125 | (970) 264-2180 | AWOS IIIP/T |